FZCom

FZ-1/10M  <--> PC MIDI Data Exchange Utility

Version 2.6

January 10, 1994

Overview:

--------

FZCom performs uploading and downloading of FZ-1/10M full, bank, voice,

effect or parameter data to or from FZ memory to a PC file over MIDI.

Parameter data is the entire set of bank, voice, and effect parameters

of a full dump WITHOUT any PCM voice data.  Parameter data can be used

to support different parameter settings of the same PCM data.  Since

the FZ assumes a parameter data transfer correctly matches the

currently loaded PCM data, it is up to the user to insure that the

correct PCM data is present in the FZ.

Because MIDI transfer of large dumps is SLOW (the better part of an hour

for 2Mbytes), transfering individual voices, parameter, and effect data is

the typical use of FZCom.  FZCom performs all appropriate handshaking with

the FZ, including retransmission of bad data packets.

System Requirements:

-------------------

PC with MPU-401 or compatible (operating in UART mode).

Microsoft compatible mouse (optional).

Release Information:

-------------------

Version 2.1 corrects failure to always place the FZ-1/10M in a DATA ERROR

state at the end of a parameter data store operation.

Version 2.2 corrects possible failure in parameter data store operation.

Version 2.3 support for non-standard MIDI port addresses.

Version 2.4 provides minor cosmetic enhancements.

Version 2.5 supports delay multipliers for running FZCom on

fast processors.

Version 2.6 supports additional command line parameters.

Command Summary:

---------------

FZCom's top-level command loop is menu based, using cursor (arrow) keys for

moving among the selections.  The highlighted selection is executed when

the carrage return key is pressed.   Each command starts with a unique

letter than can be used to directly select and execute the command using a

single keystroke.  Menus can be aborted by pressing the Esc key.  Some

commands require entry of filenames or numeric values.  These entries can

be aborted by entering a null response.

Optionally, a Microsoft compatible mouse can be used.  To use the mouse the

mouse driver must be installed before invoking FZCom.  Selections are made

by moving the mouse cursor over the desired item and clicking the left

button.  Each menu has a small dot in the upper left corner that can be

used to abort the menu (equivalent to pressing the Esc key).

The top-level commands are:

Save         (Transfers data from the FZ to a PC file)

Load         (Loads data from a PC file to the FZ)

Merge        (Merges data from a PC file with the FZ)

Verify       (Verifies data from a PC file with the FZ)

Port         (Specifies a different MIDI port base address)

New Channel  (Specifies a different MIDI transfer channel)

Delay        (Specifies a different MIDI delay multiplier)

Remote       (Toggles remote mode -- see below)

Close        (Sends a CLOSE_MIDI message to the FZ; this is useful if

the FZ transfer is hung)

Error        (Sends an ERR_MIDI message to the FZ; this is useful if

the FZ transfer is hung)

Inspect      (Display incoming MIDI data)

Quit         (Quit FZCom and return to DOS)

Using FZCom:

-----------

In order to use FZCom, the FZ must be using MIDI as its input/output

device.  To set the device to MIDI, enter the MODIFY menu on the FZ, then

select DATA_DUMP followed by SELECT_DEVICE and change the device from DISK

to MIDI.  On this same display, the following line contains the item

REMOTE MODE.  After selecting MIDI as the device, use the downarrow key to

put the cursor on REMOTE MODE.  This places the PC in charge of data

transfers.  (FZCom also supports transfers initiated by the FZ, but we'll

ignore that for now.)

To save a voice from the FZ to a PC file, enter S at the FZCom menu on the

PC.  You will be asked for the type of transfer you wish to perform (Full,

Bank, Voice, Effect, or Parameters).  Enter the first letter of the desired

transfer type (in this case V).  You will then be asked for  the bank and

voice numbers of the voice you wish to transfer.  You will also be asked

for the name of a file in which to store the data.  Be careful when

specifying the file name, if another file of the same name already exists,

it will be overwritten with the transfered data.  You need not specify a

file extension when saving data to PC files.  (You can, however, specify a

different file extension if desired.)  FZCom supplies following default

extensions according to the type of transfer:

Full       -- .FZF

Bank       -- .FZB

Voice      -- .FZV

Effect     -- .FZE

Parameters -- .FZP

Loading, merging, or verifying data from a FZCom file to the FZ is similar

to saving data.  For example, to verify the above save operation, enter V

at the FZCom menu.  You will be asked for the name of the file. In this

case, you must supply the file extension.  FZCom determines the type of

transfer (Full, Bank, Voice, Effect, or Parameters) by looking at the

contents of the file.  Again, you will be asked for the bank and voice

numbers  of the voice you wish to verify.  Verifying a file requires all

data to be uploaded to the FZ, so verification takes the same amount of

time as the save operation.  (Note that it is NOT POSSIBLE to verify a

parameters file; the FZ does not support that option.)

Manual (Non-remote) Mode:

------------------------

The above description assumes that you have placed the FZ-1/10M in REMOTE

MODE and that you are using FZCom's default remote mode assumption.  By

entering R at the FZCom menu on the PC, you toggle FZCom's assumption from

remote mode to manual mode.  (You will notice the Remote indicator in the

STATUS window changes from ON to OFF.)

When FZCom is in manual mode, the transfer must be activated at the FZ. In

this case, you supply FZCom with all information for save, load, merge, or

verify actions as in the above description.  However, FZCom does not

immediately initiate the action.  Instead, it waits for you to initiate the

action at the FZ menu.  Note that even though the FZ is not in REMOTE MODE

the device must still be set to MIDI.

The default remote mode can be changed by invoking FZCom as:

FZCom -Rn

where n = 0 specifies manual mode; n = 1 specifies remote mode (the default).

User Interface Editing Options:

------------------------------

When a value is required in an input window, the following editing commands

are available:

<ESC>       abort input operation;

<BkSp>      delete previous character;

<Ins>       toggle insert mode;

<Del>       delete current character;

leftarrow   move cursor left one character;

rightarrow  move cursor left one character;

<Return>    return the input value.

A number of the input windows in FZlist support advanced editing features.

When a default numeric value is given for an input option, the default can

be selected by simply pressing the <RETURN> key.  The numeric value can be

incremented by one using the uparrow cursor key or decremented by one using

the downarrow cursor key.  The <PgUp> key increments the value by 10, and

<PgDn> decrements the value by 10.  Finally, <Home> increments the value

to its legal maximum, and <End> to its legal minimum.

Any other key removes the default value and converts the window to  a

normal editing input window.

Non-standard MIDI Port Addresses:

--------------------------------

By default, FZCom uses a MIDI port base address of 0330 (hex).  This can

be changed by invoking FZCom as:

FZCom -Pxxxx

where xxxx is the non-standard MIDI port address (in hex).

Increasing the MIDI Delay:

-------------------------

FZCom uses an internal counter to implement low-level MIDI port delays.

By default, the multiplier used for this counter is 1 (supporting

old 8086/8088 PCs).  Faster machines require a larger multiplier value.

FZCom's delay multiplier can be changed by invoking FZCom as:

FZCom -Mn

where n is an integer.  If you are using FZCom on a fast 386/486

processor, try a delay multiplier value of 2 or 3.  If you are still

having problems try 5 or 10.  Really fast Pentiums may require

multipliers of 50 or more (up to 99).  The precise setting is not

critical; if it works, you're OK.

Note that both the MIDI port and the delay multipler can be specified (as

well as all other FZCom command line parameters):

FZCom -M2 -P033

Changing the Default MIDI Channel:

---------------------------------

By default, FZCom uses MIDI channel 1 for transfer.  This can be changed

by invoking FZCom as:

FZCom -Cn

where n is the default MIDI channel number.

Errors and Surprises:

--------------------

A Common error in using FZCom is forgetting to place the FZ into MIDI (and

REMOTE MODE) before beginning an FZCom action.  FZCom will time out with a

message that no OPEN or OK MIDI message was received.  Simply place the FZ

into the correct state and redo the operation.

Another common error is forgetting to change the FZ device back to DISK

before initiating a disk operation.  If an FZ load action shows that you

don't have any files on your disk (and you think you do), be sure to double

check that the FZ's device is set to DISK.

If a verify fails, the FZ will display a VERIFY ERROR message.  Simply

press ENTER on the FZ to clear this message and reset the FZ for another

transfer.  FZCom will time out with a message that no OPEN or OK MIDI

message was received.

It is possible to hang the FZ during a transfer by running out of disk

space during a save operation, accidently rebooting the PC during a

transfer, disconnecting MIDI cables, etc.  It is easy to see if the FZ is

hung.  It will display the EXECUTING message, but the black dot following

this message will not be moving.  Issuing a CLOSE MIDI or an ERR_MIDI command

from FZCom will often jar the FZ into an error message.  Again,

simply press ENTER on the FZ to clear this message and reset the FZ for

another transfer.

If all else fails, you may need to reboot the FZ by powering down.  Thus,

it is a good practice to always save data that took an entire evening to

create to FZ disk before using FZCom.

File Formats:

------------

FZCom duplicates the file format used by the FZ (except no file header

block is used).  The FZ format is an integral number of 1024 byte

blocks, partitioned according to the dump type (Full, Bank, Voice, or

Effect).  The source for this information is the ``CASIO Digital

Sampling Keyboard Model FZ-1 Data Structures (For Software

Developers)'' document provided by Casio.)  This reference is crucial

for anyone interfacing with FZCom files.

Voice files:

FZCom voice files (default extension .FZV) consist of a 1024 byte block

containing only voice data parameters followed by blocks containing PCM

waveform data (512 samples/block).  Thus it is easy to get at the PCM data

by skipping the first 1024 bytes of the file.  The voice data parameters

are contained in the first 192 bytes of the first block and are as follows:

-------------------------------------------------------------------------------

#define MAXE 8                          /* number of envelope steps         */

struct voicedata {

long            wavst;              /* wave start address            00 */

long            waved;              /* wave end address              04 */

long            genst;              /* generator start address       08 */

long            gened;              /* generator end address         0C */

short           loop;               /* ga mode status (see gaa)      10 */

char            loop_sus;           /* loop sustain number (0-8)     12 */

char            loop_end;           /* loop end number (0-8)         13 */

long            loopst[MAXE];       /* loop start address            14 */

/* --- b15~b12 for loop fine        */

long            looped[MAXE];       /* loop end address              34 */

/* --- b15 for jumploop flag        */

short           loopxf[MAXE];       /* loop x-feed time              54 */

unsigned short  looptm[MAXE];       /* loop time (or times)          64 */

short           dcp;                /* dcp voice pitch with detune   74 */

char            dcf;                /* frequency offset value        76 */

char            dcq;                /* filter Q offset value         77 */

char            dca_sus;            /* dca envelope sustain poshort  78 */

char            dca_end;            /* dca envelope end poshort      79 */

char            dca_rate[MAXE];     /* dca envelope rate value       7A */

unsigned char   dca_stop[MAXE];     /* dca envelope stop value       82 */

char            dcf_sus;            /* dcf envelope sustain poshort  8A */

char            dcf_end;            /* dcf envelope end poshort      8B */

char            dcf_rate[MAXE];     /* dcf envelope rate value       8C */

unsigned char   dcf_stop[MAXE];     /* dcf envelope stop value       94 */

unsigned short  lfo_delay;          /* lfo delay time                9C */

unsigned char   lfo_name;           /* lfo waveform def (MSB synch)  9E */

unsigned char   lfo_atck;           /* lfo attach value              9F */

char            lfo_rate;           /* lfo rate (time increment)     A0 */

char            lfo_dcp;            /* lfo pitch depth               A1 */

char            lfo_dca;            /* lfo amplitude depth           A2 */

char            lfo_dcf;            /* lfo filter depth              A3 */

char            lfo_dcq;            /* lfo filter Q depth            A4 */

char            vel_dcq_kf;         /* initial touch dcq follow      A5 */

char            dca_kf;             /* dca keyboard follow depth     A6 */

char            dca_rs;             /* dca note rate scaling depth   A7 */

char            dcf_kf;             /* dcf keyboard follow depth     A8 */

char            dcf_rs;             /* dcf note rate scaling depth   A9 */

char            vel_dca_kf;         /* initial touch amp kb follow   AA */

char            vel_dca_rs;         /* initial touch amp rate scale  AB */

char            vel_dcf_kf;         /* initial touch dcf kb follow   AC */

char            vel_dcf_rs;         /* initial touch dcf rate scale  AD */

unsigned char   hwid;               /* high width MIDI code          AE */

unsigned char   lwid;               /* low width MIDI code           AF */

unsigned char   cent;               /* keynote center                B0 */

unsigned char   samp;               /* sampling frequency            B1 */

char            name[14];           /* wave name                     B2 */

};                                      /* total size                    C0 */

#define VSIZE (sizeof (struct voicedata))

-------------------------------------------------------------------------------

The most important of these parameters are the first two, which specify the

number of PCM samples (not bytes) that follow in the PCM data.

Effect files:

FZCom effect files (default extension .FZE) consist of a single 1024 byte

block containing the effect parameters beginning at the 960th byte in the

block.  The effect data parameters are contained in the first 192 bytes of

the first block and are as follows:

-------------------------------------------------------------------------------

struct effectdata {

char            bend;               /* bender depth                  00 */

char            mvol;               /* master volume value           01 */

char            suss;               /* sustain switch (ON, OFF)      02 */

char            mod_lfp;            /* modulation   lfo pitch        03 */

char            mod_lfa;            /*              lfo amplitude    04 */

char            mod_lff;            /*              lfo filter       05 */

char            mod_lfq;            /*              lfo filter Q     06 */

char            mod_dca;            /*              ampl offset      07 */

char            mod_dcf;            /*              filter offset    08 */

char            mod_dcq;            /*              filter Q offset  09 */

char            fot_lfp;            /* foot volume  lfo pitch        0A */

char            fot_lfa;            /*              lfo amplitude    0B */

char            fot_lff;            /*              lfo filter       0C */

char            fot_lfq;            /*              lfo filter Q     0D */

char            fot_dca;            /*              ampl offset      0E */

char            fot_dcf;            /*              filter offset    0F */

char            fot_dcq;            /*              filter Q offset  10 */

char            aft_lfp;            /* aftertouch   lfo pitch        11 */

char            aft_lfa;            /*              lfo amplitude    12 */

char            aft_lff;            /*              lfo filter       13 */

char            aft_lfq;            /*              lfo filter Q     14 */

char            aft_dca;            /*              ampl offset      15 */

char            aft_dcf;            /*              filter offset    16 */

char            aft_dcq;            /*              filter Q offset  17 */

};                                      /* total size                    18 */

#define ESIZE (sizeof (struct effectdata))

-------------------------------------------------------------------------------

Full, Bank, and Parameter files:

FZCom Full dump file (default extension .FZF), Bank dump file (default

extension .FZB), and Parameters file (default extension .FZP) formats

contain multiple voice, bank, and (for Full and Parameters formats)

effect parameters.

A Bank dump file has one set of bank parameters (see below) starting

the first block followed by (v+3)/4 blocks of voice parameters packed 4

to a block at 256 byte offsets.  The PCM data begins at the start of

the next block (1 + ((v+3)/4)).

Here is the format of the bank parameters:

------------------------------------------------------------------------------

#define MAXV 64                         /* number of areas                  */

struct bankdata {

unsigned short  bstep;              /* bank use voice number         00 */

unsigned char   hwid[MAXV];         /* high keynote width            02 */

unsigned char   lwid[MAXV];         /* low keynote width             42 */

unsigned char   htch[MAXV];         /* high keytouch width           82 */

unsigned char   ltch[MAXV];         /* low keytouch width            C2 */

unsigned char   cent[MAXV];         /* keynote center               102 */

unsigned char   mchn[MAXV];         /* generate MIDI channel        142 */

unsigned char   gchn[MAXV];         /* generate channel select      182 */

unsigned char   bvol[MAXV];         /* generate output level        1C2 */

unsigned short  vp[MAXV];           /* voice data pointer           202 */

char            name[14];           /* bank name                    282 */

};                                      /* total size                   290 */

#define BSIZE (sizeof (struct bankdata))

------------------------------------------------------------------------------

A Full dump file begins with blocks of bank parameters, one block/bank.

The first block also contains the effect parameters, starting at the

960th byte.  The bank parameter blocks are followed by (v+3)/4 blocks

of voice parameters packed 4 to a block at 256 byte offsets.  The PCM

data begins at the start of the next block (b + ((v+3)/4)).

A Parameter dump file is identical to a Full dump file except that

the PCM data is missing.  (Actually, 1 block of PCM data is included

as a safeguard.)

All FZCom files:

All FZCom files have a special header contained in the first 1024 byte

block.  This header begins at byte 1000 and contains the following data:

-------------------------------------------------------------------------------

struct fz_file_header {

long            indicator;       /* FZCom file indicator           00 */

char            version;         /* FZCom file format version no.  04 */

unsigned char   status;          /* Type of file (F:0,B:1,V:2,E:3) 05 */

unsigned char   bn;              /* Number of banks in this file   06 */

unsigned char   vn;              /* Number of voices in this file  07 */

short           block_count;     /* Number of blocks in the file   08 */

short           wn_block_count;  /* Number of PCM data blocks      0A */

short	    unused;          /* Reserved                       0C */

} fz_file_header;                    /* total size                     10 */

-------------------------------------------------------------------------------

The 16 bytes are never used in the four FZ file formats, so placing the FZCom

header there does not affect any file data.

Disclaimers:

-----------

FZCom is provided as is, with all the usual liability disclaimers as to

correctness, suitability of purpose, etc.  The person(s) using the

software bear all risk as to the quality and performance of the

software.  In no event shall the author's liability for any damages ever

exceed the price paid for the license to use the software, regardless of

the form of the claim.

You may copy and redistribute FZCom provided the program and this

accompanying documentation are both included without modification.

FZCom is SHAREWARE/SAMPLEWARE.  IT IS NOT FREE OR PUBLIC DOMAIN.

If you paid to receive a disk containing FZCom from other than the

author or to access a network from which you downloaded FZCom, you are

NOT registered.  If you received a copy from a registered friend, you

are also NOT registered.

If you use FZCom, please register your copy by completing and returning the

enclosed registration form.

The number of FZ-1/10M units out there is small compared to ``mainstream''

instruments.  The lack of FZ-1/10M software is the result of this small

market and the complexity of the FZ.  If you are using FZ shareware

programs without registration, you are not helping to change this

situation.

Support shareware and non-copy-protected software by paying your fair

share.

Dan Corkill

9 Bristol Lane

Hadley, Massachusetts 01035

72330,226 on CompUServe, cork@cs.umass.edu on Internet.

Other Software:

--------------

FZList -- window-based FZ-1/10M parameter viewer

SHAREWARE/SAMPLEWARE ($25);

FZEdit -- window-based FZ-1/10M parameter viewer/editor --

includes all capabilities of FZList plus editing!

SHAREWARE/SAMPLEWARE ($35, $10 upgrade for registered FZList users)

-------------------------------------------------------------------------------

FZCom Registration Form

Name:    ______________________________________________________________________

Address: ______________________________________________________________________

City/State: ___________________________________________  Zipcode: _____________

FZCom Version Number: _______          Today's Date: __________________________

Registration Fee:  ____ $15

(choose one)

____ disk containing ORIGINAL samples/patches

Can I redistribute your samples/patches?

(Without charge, of course)

____ Yes (with credit)       ____ No (for my use only)

Return this form and $15 or your disk containing original FZ-1/10M

samples/patches to:

Dan Corkill

9 Bristol Lane

Hadley, Massachusetts 01035

-------------------------------------------------------------------------------

EOF