CASIO FZ DATA TRANSFERS

Jeff McClintock (jeffm@iconz.co.nz)

This text describes the structure of FZ dump data and dump procedure.

If you find errors, please let me know.

Overview.

Block Structure

BANK DATA

VOICE DATA

WAVE DATA

EFFECT DATA

PC DATA DUMPS TO AN FZ

Overview

header detail

High Speed Port

MIDI

Storing FZ data on PC disk

High Speed Port details

Overview.

A PC can send and receive (dump) data from an FZ sampling synthesizer.  The dump contains the raw sample data and the sampler's 'housekeeping' information.

Dumps contain up to 5 types of information:

1.A header,  This describes the contents and size of the data to follow. The format of the header depends on the situation. MIDI dumps have a different header than port dumps.

2.Bank data,  Contains parameters of one or more banks. e.g. which voices comprise a bank.

3.Voice data, Contains the parameters of one or more voices, e.g. envelope settings.

4.Wave data,  Actual 16 bit raw sample data.

5.Effect data, contains any FZ settings not relating directly to a voice or bank. e.g. master volume, foot pedal settings.  Effect data starts 960 bytes into a block.

Data is always sent in BLOCKS. Blocks are 1024 bytes in size.  Each block can only contain 1 type of data (bank, voice, wave etc.).  Extra information (block headers, checksum) may be 'wrapped' around each block, but is discarded as each block is received.

Example

A simple VOICE DUMP, 1 voice. All blocks are the same size except the header.  The actual sample data is spread across several WAVE blocks.  The last block is padded out to 1024 bytes size.  Blocks are sent in this order.

[HEADER     ]

[VOICE BLOCK]

[WAVE  BLOCK]

[WAVE  BLOCK]

[WAVE  BLOCK]

[WAVE  BLOCK] 'total of 4096 bytes sample data (2048 samples)

A full dump

[HEADER     ]

[BANK  BLOCK] ' 1st bank

[BANK  BLOCK] ' 2nd bank

[VOICE BLOCK] ' 4 voices

[VOICE BLOCK] ' 4 more voices

[WAVE  BLOCK]

[WAVE  BLOCK]

[WAVE  BLOCK]

[WAVE  BLOCK]

[WAVE  BLOCK] 'sample data. 8 voices worth. Concatenated

an effect dump

[HEADER	]

[EFFECT BLOCK]     Effect data starts 960 bytes into the block.

Block Structure

BANK DATA

Each bank uses 656 bytes of data leaving the remainder of the block unused.

This is the data accessed by [EDIT BANK] on the FZ.

Here is the layout of the data in a 'C' language format.  Beware, your C compiler may have different size 'short', 'int', or 'long' data types.

#define MAXV 64		// Maximum number of voices

unsigned int   bstep;            	// 0-64 Number of voices used in the bank

unsigned short hwid[MAXV];	// Hi-Note settings (MIDI note number)

unsigned short lwid[MAXV];	// Lo-Note settings (MIDI note number)

unsigned short htc[MAXV];	// 1-127 Velocity split hi

unsigned short ltch[MAXV];	// 1-127 Velocity split lo

unsigned short cent[MAXV];	// Original note (MIDI note number)

unsigned short mchn[MAXV];	// 0-15 Recieve MIDI channel

unsigned short gchn[MAXV];	// Each bit represents an 'individual output'

unsigned short bvol[MAXV];	// 1-127 Area volume (Usually 127)

unsigned int   vp[MAXV];		// 0-63 Voice number to use in area

char           name[14];		// Bank name, last two bytes should always be 0

What does this all mean?;

short	- A binary value stored in 1 byte

int	- A binary value stored in 2 bytes

long	- A binary value stored in 4 bytes

unsigned	- only positive values

char	- ASCII codes

[MAXV]- means that there are 64 of each piece of data.

//	- preceedes a comment

VOICE DATA

Each voice uses 192 bytes of data.  Each VOICE block can contain 1 to 4 voices.  When a block contains more than 1 voice the voices are aligned on 256 byte boundaries. eg voice 1 data starts at byte 0 and ends at byte 191, voice 2 data starts at byte 256, and ends at 447.

All parameters default to 0 unless otherwise stated.

long    wavst;		// Sample data start address

long    waved;		// Sample data end address

long    genst;		// Sample play start address

long    gened;		// Sample play end address (usually =waved - 2)

int     loop;		// 0000h No waveform,

// 01d7h Normal sound,(default)

// 101dh Reversed,

// 2014h Cueing sound (played with WHEEL),

// 0013h Synthesised waveform.

// FZ supports up to 8 loops

short    loop_sus;		// 0-8 sustain loop, 8 = No sustain loop

short    loop_end;		// 0-8 last loop, 8 = execute all loops

long    loopst[8];		// Loop starts, upper 8 bits are loop fine settings (set =gened for unused loop)

long    looped[8];		// Loop ends, MSB = Skip or Trace (set =gened for unused loop)

int     loopxf[8];		// 0-1023 Loop cross fade time

unsigned int    looptm[8];	// 1-1022 Loop time 16 ms to 16 s (default is 64), 1024 = continuous loop,

//                                                                                  1023 = loop during sustain

int     dcp;		// 0-255 loop pitch correction times 1/256 semitone

short    dcf;		// 0-127 Filter cut off frequency

short    dcq;		// 0-127 Filter resonance (Q) lower 3 bits ignored

// The FZ has 8 stage envelopes (rate/level type)

// A good default is to set all dca_rate[] to 0xc0, and dcf_rate[] to 0x90

except for dca_rate[0]=0x7f,dcf_rate[0]=0x7f,

dcf_stop[0] to 0xff , dca_stop[0] to 0xff

short    dca_sus;		// 0-7 Sustain section # on DCA envelope

short    dca_end;		// 0-7 End section # on DCA envelope (default to 7)

short    dca_rate[8];	// 0-127 Envelope rates MSB = direction up or down

unsigned short   dca_stop[8];	// 0-255 Envelope section end level

short    dcf_sus;			// 0-7 Sustain section # on DCF envelope

short    dcf_end;			// 0-7 End section # on DCF envelope (default to 7)

short    dcf_rate[8];		// 0-127 Envelope rates MSB = direction up or down

unsigned short   dcf_stop[8];	// 0-255 Envelope section end level

unsigned int    lfo_delay;		// 0-65535 LFO Delay time 2 ms steps

unsigned short   lfo_name;		// 0-Sine, 1-Saw up, 2-Saw down, 3-triangle,4-rectangle, 5 random. MSB=phase sync (default MSB =1)

unsigned short   lfo_atck;	// 1-127

short    lfo_rate;		// 0-127 (default to 0x40)

short    lfo_dcp;		// 0-127 LFO effect on Pitch

short    lfo_dca;		// 0-127 LFO effect on amplitute

short    lfo_dcf;		// 0-127 LFO effect on filter

short    lfo_deq;		// 0-127 LFO effect on filter Q

short    vel_deq_kf;	// -127 to 127 ,velocity effect on filter Q

short    dca_kf;		// DCA amplitude Key follow

short    dca_rs;		// DCA rate Key follow

short    dcf_kf;		// DCF amplitude Key follow

short    dcf_rs;		// DCF rate Key follow

short    vel_dca_kf;	// -127 to 127 Key Velocity effect on DCA envelope amplitude (default to 0x30)

short    vel_dca_rs;	// -127 to 127 Key Velocity effect on DCA envelope rate

short    vel_dcf_kf;	// Velocity effect on filter envelope amplitude

short    vel_dcf_rs;	// Velocity effect on filter envelope rate

unsigned short   hwid;	// Hi note (MIDI note number) (default to 0x60)

unsigned short   lwid;	// Low note (MIDI note number)(default to 0x24)

unsigned short   cent;	// Original note (MIDI note number)(default to 0x48)

unsigned short   samp;	// sampling freq 0 - 36khz, 1 - 18khz, 2 - 9khz

char    name[14];		// ASCII name. last 2 bytes always 0

EFFECT DATA

struct effectdata

{

short	bend;		// bender depth

short	mvol;		// master volume

short	suss;		// sustain switch ON/OFF

short	mod_lfp;		// modulation to	lfo pitch

short	mod_lfa;		//		lfa amp

short	mod_lff;		//		lfo filter

short	mod_lfq;		//		lfo filter q

short	mod_dcf;	//		filter offset

short	mod_dca;	//		amp offset

short	mod_dcq;	//		filter q offset

short	fot_lfp;		// foot volume to..

short	fot_lfa;		//

short	fot_lff;		//

short	fot_lfq;		//

short	fot_dca;		//

short	fot_;dcf		//

short	fot_;dcq		//

short	aft_lpf;		// after touch to..

short	aft_lfa;		//

short	aft_lff;		//

short	aft_lfq;		//

short	aft_dca;		//

short	aft_dcf;		//

short	aft_dcq;		//

};

WAVE DATA

This is simply 16 bit sample data, signed.

PC DATA DUMPS TO AN FZ

Overview

1 - PC sends 'REMOTE CODE'   (prepare for transfer), can be LOAD, SAVE, MERGE, VERIFY

2 - PC waits 1 second

3 - PC Sends 'OPEN CODE'     This dump header describes the data blocks

3 - PC Sends data blocks

This assumes the FZ is set to [REMOTE MODE], you can skip step 1 and 2 if you

use [LOAD VOICE] on the FZ.

When using MIDI, there is a handshaking protocol to ensure all data arrives ok.  The FZ acknowledges each packet of data.  With the high speed port, there is only low-level hardware handshaking.

header detail

High Speed Port

REMOTE CODE (PORT)

17 bytes of data

[7F][][][][][][eb][ev][sta][mod][][][][][][][sum]

OPEN CODE (PORT)

17 bytes of data

[sta][1L][1H][Bn][Vn][WL][WH][eb][ev][][][][][][][][sum]

[7F]	- one byte of data, hexadecimal value 7F (127 decimal)

[]	- one byte of data, doesn't matter what (typically 0)

[eb]	- The bank number to dump ie 0-7, 0x7f means dump current bank

[ev]	- The voice number to dump ie 0-63, 0x7f means dump current voice

[sta]	- Type of dump, 0 - FULL, 1 - VOICE, 2 - BANK, 3 - EFFECT

[mod]	- 0 - SAVE, 1 - LOAD, 2 - MERGE, 3 - VERIFY

[sum]	- Checksum, Add up total of all bytes, logical AND with 0xff

(255), subtract from 0x100 (256)

'C' language example:  checksum = 256 - (checksum & 255);

[1L][1H]- Total number of blocks to follow, split into high and low bytes

[Bn]	- Number of banks

[Vn]	- Number of voices

[WL][WH] - Total number of blocks of wave data (samples), split into high and low bytes

When sending an effect dump, Bn, Vn, and WL/WH are all zero ( 0 ).

Data is then sent in blocks (1024 bytes) with the addition of a checksum at the end of each block.

MIDI

When transmitting data over MIDI, each block is further divided into packets.Each packet sent to the FZ over MIDI has a SYSEX HEADER, and an end-of-sysex byte.

The Casio FZ MIDI PACKET HEADER

[F0][44][02][00][7n]

7n - the 'n' represents the midi channel. 0=chan 1.

The end of each packet must have [F7].  Each packet of data is acknowledged by a message from the FZ;

REMOTE CODE (MIDI)

11 bytes of data (shown in hexadecimal)

[F0][44][02][00][7n][7F][eb][ev][sta][mod][F7]

OPEN CODE (MIDI)

16 bytes of data

[F0][44][02][00][7n][70][sta][Bn][Vn][W0][W1][W2][W3][eb][ev][F7]

These contains much the same info as the PORT codes. Note that 2-byte values have to be split into 4 nybles (4 bits each).

Each packet of data is then sent, 64 bytes of data at time.  Each byte is split into nybles when sent over midi.  So 64 bytes of data = 128 bytes over MIDI.

[F0][44][02][00][7n][74][ 128 bytes of data][checksum][F7]

To calculate checksum. Add up the total of all data bytes, subtract from 256 then logical 'AND' with 127.

Example in 'C'

checksum &= 255;

checksum = ( 256 - checksum ) & 127;

Each time the FZ receives a packet, it replies

[F0][44][02][00][7n][72][F7]

, or if there was an error

[F0][44][02][00][7n][73][F7] (second to last byte changes).

Once the entire dump is complete, the FZ sends a ‘Close’ command

[F0][44][02][00][7n][71][F7]

Storing FZ data on PC disk

FZDUMP stores data on disk much the same as it is received. The data is stored in blocks of 1024 bytes.  The header describing the data is stored starting at 1000 bytes into the file (taking advantage of an unused section of the first block).  The file has the extension .FZF (Full dump), .FZB (Bank dump), or .FZV (Voice Dump).  The file extension is redundant, as the file header also indicates a full dump (bn > 1), bank dump (bn = 1) or voice dump (bn = 0).

struct fz_file_header {

long            indicator;              /* FZCom file indicator     (always =  476549224)    00 */

short           version;                /* FZCom file format version no.  (always = 1) 04 */

unsigned char   status;          /* Type of file (Full:0, Voice:1 Bank:2, ,Effects:3) 05 */

unsigned char   bn;                /* Number of banks in this file   06 */

unsigned char   vn;                /* Number of voices in this file  07 */

unsigned char   dum2;           /* unused? (set to 0)*/

short           block_count;          /* Number of blocks in the file  (1 block = 1024 bytes)  08 */

short           wn_block_count;  /* Number of PCM data blocks      0A */

short	    unused;                       /* Reserved         (set to 0)              0C */

};

# Casio FZ to PC High Speed Port - Low Level Details

## PC and FZ pin-outs

Note: I am using some PC pins for different purposes than intended (standard use in brackets)

FZ High-Speed Port						       PC Parallel Port

DATA0	1					 2 DATA0

DATA1	3					 3 DATA1

DATA2	5					 4 DATA2

DATA3	7					 5 DATA3

etc						   etc

DATA7	15					 9 DATA7

STB	19					 1 STROBE

STBO	22					10 STROBE (ACK)

ACK	21					16 ACKO (INIT)

ACKO

BUSY	24					17 BUSYO (SEL)

BUSYO	25					11 BUSY

GND   2					25 GROUND

## FZ to FZ cable pin-outs. DO NOT EVER CONNECT THIS TO A PC

FZ High-Speed Port						      FZ High-Speed Port

DATA0	1					 1 DATA0

DATA1	3					 3 DATA1

DATA2	5					 5 DATA2

DATA3	7					 7 DATA3

etc						   etc

DATA7	15					15 DATA7

STB	19					22 STROBE

STBO	22					19 STROBE (ACK)

ACK	21					 8 ACKO (INIT)

ACKO   8                            21

BUSY	24					25 BUSYO (SEL)

BUSYO	25					24 BUSY

GND   2					 2 GROUND

For general information on programming the PC parallel port see ’Interfacing the IBM PC Parallel Printer Port - Zhahai Stewart’ (Available on the WWW)

Signals.

| Signal line | PC Pin | Register bit | Notes |
| --- | --- | --- | --- |
| TRISTATE | NA | C4 | Sets PC Port to input mode, all data pins enter a high-impedance state. This only works on bi-directional ports. You must not connect an FZ to the PC unless the port is tri-stated. If you do not understand this point, DON’T try it. |
| BUSYO | 17 | C3 (inverted) | Indicates PC is not ready to receive data |
| ACKO | 16 | C2 (inverted) | Set FZ’s direction of data transfer ( ACK low sets FZ to tri-state mode) |
| STROBEO | 1 | C0 | Pulsing this tells FZ to accept 1 data byte |
| BUSY | 11 | S7 | Tells PC, FZ has accepted a byte |
| STROBE | 10 | S6 | Tells PC to accept 1 byte (Via an interrupt). |

### Low level data transfer details

Here is a brief overview of how to transfer data to an FZ.  The timing is very critical, you must use an interrupt routine to receive data. You must disable all interrupts when sending data.

Because the FZ was not designed to talk to a PC, some of it’s signals are inverted from what you would expect (eg the strobe line).  So the PC interrupt is triggered on the trailing edge of the strobe signal, not the leading edge.  This means you don’t get the first byte of any data transfer to the PC (you get a 0 byte instead).  Each block of data from the FZ has a checksum.  You can use this to recover the ‘lost’ first byte.

If both the FZ and the PC try to send data at the same time, you will destroy your PC or FZ port (or both).  A better way would be to make some type of buffer circuit to prevent this (don’t ask me how).  I have tried various resistors in the data lines, but they make the transfer unreliable.  I now use no resistors (I MUST unplug the cable when not running FZ-DUMP).

For an example of how to do this see FZ-DUMP for DOS source code at:

www.iconz.co.nz/~jeffm/fzdump.htm

The relevant files are ‘port_io.cpp’ for the low level stuff, and ‘fz_port.cpp’ for the high level stuff.

### Preparing PC to receive data

prepare PC port for input, and FZ port for output:

TRISTATE data lines

raise ACK

clear FZ’s output buffer

lower BUSY

small time delay (approx. 2 milli second )

raise BUSY

### Sequence of events receiving 1 byte FZ to PC

PC lowers BUSY

FZ Places DATA on outputs

FZ pulses STROBEOUT ( 0.25 micro sec )

PC raises BUSY

PC reads DATA off  inputs

### Preparing PC to receive data

lower ACK

lower BUSY

lower STROBE

# Sequence of events sending 1 byte PC to FZ

PC Places DATA on outputs

PC raises BUSYO

PC raises STROBO

PC waits for FZ to raise BUSY

FZ reads DATA off  inputs

PC lowers STROBEO

PC lowers BUSY

PC waits for FZ to lower BUSY

Written by Jeff McClintock

Last Updated : 16/5/1999

Disclaimer:

This may have errors.  If you connect your FZ to your PC wrongly, you might destroy it.